|  | Simulation of FullAssembly Date: Thursday, November 21, 2024 Designer: Vittorio Pastore Study name: Moment Arm Analysis type: Static. Table of Contents Description 1 Study Properties 2 Units 2 Material Properties 3 Loads and Fixtures 4 Connector Definitions 4 Interaction Information 5 Mesh information 6 Sensor Details 7 Resultant Forces 8 Beams 8 Study Results 9 Conclusion 11. |
| --- | --- |
| # Description No Data |  |

# Study Properties Study name; Moment Arm. Analysis type; Static. Mesh type; Solid Mesh. Thermal Effect:; On. Thermal option; Include temperature loads. Zero strain temperature; 298 Kelvin. Include fluid pressure effects from SOLIDWORKS Flow Simulation; Off. Solver type; Automatic. Inplane Effect:; Off. Soft Spring:; Off. Inertial Relief:; Off. Incompatible bonding options; Automatic. Large displacement; Off. Compute free body forces; On. Friction; Off. Use Adaptive Method:; Off. Result folder; SOLIDWORKS document (C:\Users\super\OneDrive\Documents\!Pastore\KinovaArmTS\CAD).

# Units Unit system:; SI (MKS). Length/Displacement; mm. Temperature; Kelvin. Angular velocity; Rad/sec. Pressure/Stress; N/m^2.

# Material Properties Model Reference; Properties; Components. Name:; 6061 Alloy. Model type:; Linear Elastic Isotropic. Default failure criterion:; Max von Mises Stress. Yield strength:; 5.51485e+07 N/m^2. Tensile strength:; 1.24084e+08 N/m^2. Elastic modulus:; 6.9e+10 N/m^2. Poisson's ratio:; 0.33. Mass density:; 2,700 kg/m^3. Shear modulus:; 2.6e+10 N/m^2. Thermal expansion coefficient:; 2.4e-05 /Kelvin.; SolidBody 1(Cut-Extrude2)(8982K546_Multipurpose 6061 Aluminum 90 Degree Angle Machined-1), SolidBody 1(Cut-Extrude2)(8982K546_Multipurpose 6061 Aluminum 90 Degree Angle Machined-2), SolidBody 1(Chamfer1)(BaseMount-1), SolidBody 1(Cut-Extrude2)(MiddlePiece-1), SolidBody 1(Cut-Extrude2)(MiddlePiece-2). Curve Data:N/A.

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 6 face(s). Type:; Fixed Geometry.. Resultant Forces Components; X; Y; Z; Resultant. Reaction force(N); -1.98185e-06; 9.20147e-06; -49.9999; 49.9999. Reaction Moment(N.m); 0; 0; 0; 0.. Load name; Load Image; Load Details. Remote Load/Mass (Rigid connection)-1; Entities:; 1 face(s). Connection Type:; Rigid. Coordinate System:; Global cartesian coordinates. Translational Components:; ---,---,50 N. Rotational Components:; ---,---,---. Reference coordinates:; 0 150 0 mm..

# Connector Definitions No Data

# Interaction Information Interaction; Interaction Image; Interaction Properties. Global Interaction; Type:; Bonded. Components:; 1 component(s). Options:; Independent mesh..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Blended curvature-based mesh. Jacobian points for High quality mesh; 16 Points. Maximum element size; 4.02154 mm. Minimum element size; 0.303755 mm. Mesh Quality; High. Remesh failed parts independently; Off. Reuse mesh for identical parts in an assembly (Blended curvature-based mesher only); Off. ## Mesh information - Details Total Nodes; 139813. Total Elements; 74164. Maximum Aspect Ratio; 5.4497. % of elements with Aspect Ratio < 3; 95.9. Percentage of elements with Aspect Ratio > 10; 0. Percentage of distorted elements; 0. Time to complete mesh(hh;mm;ss):; 00:00:04. Computer name:; VITTOPC. ## Mesh Quality Plots Name; Type; Min; Max. Quality1; Mesh; -; -. FullAssembly-Moment Arm-Quality-Quality1.

# Sensor Details No Data

# Resultant Forces ## Reaction forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; -1.98185e-06; 9.20147e-06; -49.9999; 49.9999. ## Reaction Moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 0.

## Free body forces Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N; 0.000109967; 0.000125697; 8.07643e-06; 0.000167205. ## Free body moments Selection set; Units; Sum X; Sum Y; Sum Z; Resultant. Entire Model; N.m; 0; 0; 0; 1e-33.

# Beams No Data

# Study Results Name; Type; Min; Max. Stress1; VON: von Mises Stress; 1.435e-02N/m^2 Node: 60156; 5.139e+07N/m^2 Node: 90497. FullAssembly-Moment Arm-Stress-Stress1. Name; Type; Min; Max. Displacement1; URES: Resultant Displacement; 0.000e+00mm Node: 2; 1.046e-02mm Node: 103294. FullAssembly-Moment Arm-Displacement-Displacement1. Name; Type; Min; Max. Strain1; ESTRN: Equivalent Strain; 3.270e-13 Element: 39036; 3.759e-04 Element: 50936. FullAssembly-Moment Arm-Strain-Strain1.

# Conclusion